=====================================================================


===                                                               ===


===                      VWGIS-CD ACCURACY                        ===


===                    LAST UPDATE:  4/15/98                      ===


===                                                               ===     ===	Provided by: Merced County Association of Governments     	===


===           369 West 18th Street - Merced, CA - 95340    		===


===                                                               ===


===     For more information, please contact Robert Beckler or    ===


===     Richard Green at (209)723-3153 or FAX# (209)723-0322.     ===


===                                                               ===


===                                                               ===


=====================================================================


=====================================================================


The positional accuracy ranges for +/- 40' on most of the data


sets contained on this CD.  Some data sets may be more or less


accurate depending on the source material.





The first step to completing these data sets was to create a 


geo-referenced control network.  The roads and Public Land Survey


Lines were traced from USGS 7.5 minute topo quad maps.  These maps


claim to have an accuracy of approximately +/- 40'.  The exceptions


to this were Merced County and Kern County.  Merced traced a GPS


control grid and the roads from aerial photography.  This has an


accuracy level of approximately +/- 10'.  Kern County has a network


of GPS points that were driven as well as constructed from survey


maps.  This has an accuracy of +/- 5'.





The parcel maps (suffixed with _pcl) were traced from Assessor's


Parcel Maps with scales ranging from 1" = 40' to 1" = 1 mile.  These


were then placed and edgematched to fit within the framework of the


above mentioned roads files.





The General Plan & Zoning maps included on this CD were 


constructed using the parcel map as a base and following parcel


lines exactly when possible.  When necessary, the maps were scanned


and traced to fit the areas that didn't follow parcel lines.





Data sets that were provided by outside agencies have


various levels of accuracy.  When possible, the accuracy of the 


data will be listed in the file metadata.txt included on this CD.


=====================================================================





